# Alaska Department of Environmental Conservation

# Air Permit Program

# Review of

# Donlin Gold LLC’s Ambient Demonstration

# for the

# Donlin Gold Project

# Construction Permit AQ0934CPT02

Prepared by: James Renovatio Month DD, 2022

\\jn-svrfile\groups\AQ\PERMITS\AIRFACS\Donlin\Construct\CPT02\Pre\AQ0934CPT02 Modeling Review.docx

Table of Contents

1.	INTRODUCTION	1

2.	REPORT OUTLINE	1

3.	BACKGROUND	1

3.1.	Project Location and Area Classification	2

3.2.	Project Classification	2

3.3.	Ambient Demonstration Requirements	2

3.4.	Modeling Protocol Submittal	3

3.5.	Application Submittal and Amendments	4

3.6.	PM-2.5 Minor Source Baseline Date	4

4.	PRE-CONSTRUCTION MONITORING DATA	4

5.	SOURCE IMPACT ANALYSIS	7

5.1.	Approach	7

5.2.	Model Selection	7

5.3.	Meteorological Data	7

5.4.	Coordinate System	9

5.5.	Terrain	9

5.6.	EU Inventory	10

5.7.	Emission Rates	11

5.8.	Point Source Parameters	15

5.9.	Open Pit Parameters	17

5.10.	Volume Source Parameters	18

5.11.	Area Source Parameters	18

5.12.	Pollutant Specific Considerations	19

5.13.	Downwash	22

5.14.	Ambient Air Boundary	23

5.15.	Worker Housing	24

5.16.	Receptor Grid	24

5.17.	Off-Site Impacts	24

5.18.	Design Concentrations	25

5.19.	Results and Discussion	26

6.	OZONE IMPACTS	28

7.	ADDITIONAL IMPACT ANALYSES	29

7.1.	Visibility Impacts	29

7.2.	Soil and Vegetation Impacts	30

7.3.	Associated Growth Analysis	30

8.	CONCLUSION	31

INTRODUCTION

This report summarizes the Alaska Department of Environmental Conservation’s (Department’s) findings regarding the ambient analysis submitted by Donlin Gold, LLC (Donlin) for the Donlin Gold Project. Donlin submitted this analysis in support of their 29 October, 2021 application for a Prevention of Significant Deterioration (PSD) permit (AQ0934CPT02). The Donlin Gold project triggers PSD review for oxides of nitrogen (NOx), carbon monoxide (CO), total particulate matter (PM), particulate matter with an aerodynamic diameter of 10 microns or less (PM-10), particulate matter with an aerodynamic diameter of 2.5 microns or less (PM-2.5), volatile organic compounds (VOC), and greenhouse gases (GHG).

Donlin provided the source impact analysis required under 40 CFR 52.21(k), pre-construction monitoring analysis required under 40 CFR 52.21(m)(1), and additional impact analysis required under 40 CFR 52.21(o). They demonstrated that operating the project emissions units (EUs) within the restrictions listed in this report will not cause or contribute to a violation of the following Alaska Ambient Air Quality Standards (AAAQS) listed in 18 AAC 50.010: one-hour and annually averaged nitrogen dioxide (NO2); one-hour and eight-hour carbon monoxide (CO); 24-hour particulate matter with an aerodynamic diameter of 10 microns or less (PM-10); 24-hour and annually averaged particulate matter with an aerodynamic diameter of 2.5 microns or less (PM-2.5); and eight-hour ozone (O3). Donlin also demonstrated that the project impacts will not cause or contribute to a violation of the following Class II maximum allowable increases (increments) listed in 18 AAC 50.020: annually averaged NO2, 24-hour PM-10, annually averaged PM-10, 24-hour PM-2.5, and annually averaged PM-2.5.

REPORT OUTLINE

The Department’s findings regarding Donlin’s approach to meet the pre-construction monitoring requirement under 40 CFR 52.21(m) is described in Section 4. The Department’s findings regarding the additional impact analysis under 40 CFR 52.21(o) is described in Section 7.

Donlin used a multifaceted approach to address the ambient demonstration requirements under 40 CFR 52.21(k). They used computer analysis (modeling) to predict the NO2, CO, PM-10, and direct PM-2.5 air quality impacts; ambient data and qualitative discussion to characterize the existing secondary PM-2.5 impacts; and a qualitative approach to address the ambient O3 and project-related secondary PM-2.5 impacts. The Department’s findings regarding Donlin’s NO2, CO, PM-10 and PM-2.5 assessments are in Section 5. Its findings regarding their qualitative O3 analysis is in Section 6.

BACKGROUND

Donlin is proposing to construct and operate an open-pit gold mine. Salient features of the proposed construction include: tailings and waste rock facilities; a process plant with a nominal production rate of 59,000 short tons (tons) of ore per day; a 220 megawatt (MW) power plant; and various supporting facilities.

The Department issued Air Quality Control Construction Permit AQ0934CPT01 to Donlin on 30 September, 2017. They are authorized to operate under this active permit, though current application materials indicate that Donlin has yet to commence construction of the stationary source. AQ0934CPT01 will be rescinded and replaced with AQ0934CPT02 upon issuance of the latter. Previous permitting efforts include an owner requested limit (ORL) authorization AQ0934ORL01, which was rescinded following the Department’s issuance of AQ0934CPT01.

Additional information regarding Donlin’s proposed project, its triggered permit classifications, and the ambient demonstration requirements for those classifications are discussed subsequently.

Project Location and Area Classification

The stationary source will be situated within the Kuskokwim Mountain region of western Alaska, approximately 450 kilometers (km) west of Anchorage and 15 km north of the community of Crooked Creek. This area is unclassified in regard to compliance with the AAAQS. It is located within a Class II area of the South Central Alaska Intrastate Air Quality Control Region for the purpose of compliance with increment. The project is approximately 315 km from the nearest Class I area, Denali National Park (Denali).

Project Classification

Donlin’s project triggers PSD review under 18 AAC 50.306 for the pollutants and regulatory requirements discussed under the Introduction section of this report. Their project is also subject to the requirements for minor permits classified under:

- 18 AAC 50.502(b)(3), for rock crushers with a rated capacity of five tons-per-hour (tph) or greater;

- 18 AAC 50.508(5), for ORLs to avoid

  - PSD review for the emissions of sulfur dioxide (SO2); and

  - classification as a major source of hazardous air pollutants; and

- 18 AAC 50.508(6), for their request to revise/rescind AQ0934CPT01.

The provisions of the minor permit will be issued as a part of the PSD permit under 18 AAC 50.502(a)(1).

Ambient Demonstration Requirements

The State of Alaska’s PSD requirements are described in 18 AAC 50.306. PSD applicants must essentially comply with the federal PSD requirements in 40 CFR 52.21. Except as noted in 40 CFR 52.21(i), the ambient requirements include:

- A Source Impact Analysis, i.e., an ambient demonstration for the PSD-triggered pollutants with an associated ambient air quality standard or increment, per 40 CFR 52.21(k);

- An Air Quality Analysis, i.e., pre-construction monitoring data, for the PSD-triggered pollutants with an associated ambient air quality standard or increment, per 40 CFR 52.21(m);

- An Additional Impact Analysis per 40 CFR 52.21(o); and

- A Class I Impact Analysis, for stationary sources that may affect a Class I area, per 40 CFR 52.21(p).

Donlin’s project is sufficiently distant from Denali that it does not warrant a Class I Impact Analysis. The Department, nevertheless, notified the National Park Service (NPS) about the proposed project by e-mail on 23 November, 2021. It articulated an understanding that the NPS would not be requesting a Class I assessment under 40 CFR 52.21(p) unless requested. The NPS provided tacit approval of the Department’s understanding by not issuing a response. The Department notes that Donlin’s proposed project and approach to demonstrate ambient impacts has not meaningfully changed from that submitted in support of AQ0934CPT01. Department staff, therefore, continue to rely upon their previous findings as appropriate; see the modeling report for said permit for detail.

Applicants subject to the requirements for permits classified under 18 AAC 50.502(b) do not need to submit an ambient demonstration unless requested by the Department in comport with 18 AAC 50.540(c)(2)(D). The Department did not request a demonstration under the former since Donlin’s application contemporaneously triggered multi-pollutant modeling requirements in association with its PSD classification. There are no ambient air demonstration requirements associated with their classification under 18 AAC 50.508(5).

The recission of AQ0934CPT01 warrants classification under 18 AAC 50.508(6). This classification requires an evaluation of the potential impacts to an underlying ambient demonstration under 18 AAC 50.540(k)(3)(C). Donlin met this requirement by submitting a revised ambient demonstration with their application materials.

Modeling Protocol Submittal

Donlin did not provide a modeling protocol in advance of their 29 October, 2021 application for AQ0934CPT02. The Department notes, however, that they previously provided a protocol in support of their application for AQ0934CPT01 that remains representative of their current approach. The Department, therefore, continues to rely upon its review of Donlin’s 10 July, 2015 modeling protocol as it relates to key aspects of their modeled approach for AQ0934CPT02.

Application Submittal and Amendments

Donlin submitted an application for AQ0934CPT02 to the Department on 29 October, 2021. They provided three supplemental submissions by e-mail on the 9th of March, 24th of March, and 6th of May, 2022. The supplemental submissions were prepared in response to Department requests for additional information and generally address the PSD requirements under 40 C.F.R. 52.21(j) and discrete EU emissions assumptions.

PM-2.5 Minor Source Baseline Date

Donlin’s application for PSD Construction Permit AQ0934CPT01 established a PM-2.5 minor source baseline date for the South Central Alaska Intrastate Air Quality Control Region. This date is 15 October, 2015, or the date the Department found said application to be complete as noted in Table 2 of 18 AAC 50.015. Subsequent increases in minor source PM-2.5 emissions within the region will therefore be increment consuming.

PRE-CONSTRUCTION MONITORING DATA

40 CFR 52.21(m)(1) requires PSD applicants to provide ambient air monitoring data that characterizes existing air quality in the vicinity of the proposed project. The requirement is applicable to pollutants that are subject to PSD review and have a National Ambient Air Quality Standard (NAAQS). Existing pollutant concentrations or project impacts less than the applicable Significant Monitoring Concentrations (SMCs) in 40 CFR 52.21(i)(5) do not warrant submission of these data. For those pollutants where monitoring is required, the data are to be collected prior to construction, i.e. as pre-construction monitoring. An applicant may separately use monitoring data to characterize natural and anthropogenic impacts in the project area that are not readily defined through dispersion modeling, i.e. background data. Ambient monitoring data used to characterize background impacts are intended to better estimate the cumulative impacts from a proposed project and are distinct from the requirements for pre-construction monitoring; see Section 5.17 for detail on Donlin’s use of background data.

Pre-construction monitoring data must be collected at a location and in a manner that is consistent with the U.S. Environmental Protection Agency’s (EPA’s) Ambient Monitoring Guidelines for Prevention of Significant Deterioration (EPA-450/4-87-007), which the Department has adopted by reference in 18 AAC 50.035(a)(5). In summary, the data must be collected at the location(s) of existing and proposed maximum impacts, current, and meet the PSD quality assurance requirements described in 18 AAC 50.215(a).

Donlin met their pre-construction monitoring requirements by collecting 12 or more months of PSD-quality ambient data for all PSD-triggered pollutants with a NAAQS. They collected the pollutant data at the New Air Station (NAS) monitoring site, situated approximately 300 meters (m) southeast of their exploration camp site; see Figure 1 for detail.

A notable, but immaterial exception to Donlin’s aforementioned data collection effort is germane to their PM-10 pre-construction monitoring. Donlin collected the first two months of their July 2006 through June 2007 PM-10 data at the ‘Camp’ meteorological station. They subsequently relocated the PM-10 monitor to the NAS site due to expansion of their exploration camp. The Department conducted a site visit in September of 2006 and determined that the relocation did not warrant a restart of the monitoring period.

The date of initial data collection and duration of monitoring period varied by pollutant. Donlin prepared multiple Quality Assurance Project Plans (QAPPs) for Department review and approval to ensure an appropriate approach for obtaining ambient data was observed. They also submitted the subsequent data sets for Department review and approval. Table 1 provides a pollutant-specific summary of the resulting periods with PSD-quality data.

Table 1. Pre-Construction Monitoring Summary

| Pollutant | Avg. Period | Monitoring Period(s) | Max. Conc. (µg/m3) | AAAQS (µg/m3) | % of AAAQS |
| --- | --- | --- | --- | --- | --- |
| NO2 | One-hour | Nov. 2006 – Nov. 2007; Jan. – Dec. 2008; Dec. 2010 – Nov. 2011; April 2012 – April 2013 | 21 | 188 | 11 |
|  | Annual |  | 1.4 | 100 | 2 |
| CO | One-hour | Nov. 2006 – Nov. 2007; Jan. – Dec. 2008 | 687 | 40,000 | 2 |
|  | Eight-hour |  | 458 | 10,000 | 5 |
| O3 | Eight-hour | Dec. 2010 – Nov. 2011; April 2012 – April 2013 | 100 | 140 | 71 |
| PM-10 | 24-hour | July 2006 – June 2007; July 2007 – June 2008 | 14 | 150 | 9 |
| PM-2.5 | 24-hour | Jan. – Dec. 2008 | 6.8 | 35 | 19 |
|  | Annual |  | 2.3 | 12 | 19 |

Table Note: Some values are different from those provided by Donlin in their application materials. These slight differences are due to variation in rounding practices when converting values from a volumetric basis to a mass basis. None of the differences are substantive, nor do they alter the conclusion that the measured concentrations currently demonstrate compliance with the AAAQS.

The AAAQS and maximum concentrations presented in Table 1 are measured according to the form of the given AAAQS. The Department notes that it is reporting the gaseous pollutants on a mass basis in micrograms per cubic meter (µg/m3), which is the convention used in modeling, rather than on a volumetric basis, e.g. in parts-per-million (ppm), which is the convention typically used in monitoring reports. Particulate pollutants are measured and reported on a mass basis.

SOURCE IMPACT ANALYSIS

Donlin performed a cumulative modeling analysis to estimate their NO2, CO, PM-10 and direct PM-2.5 impacts. The various aspects of their analysis are discussed below.

Approach

Donlin performed multiple sets of model runs for each pollutant and averaging period in support of their 29 October, 2021 PSD application. One group of runs included a full receptor grid, described in Section 5.16, and a modeled characterization of merged exhaust plumes from the primary power plant engines, discussed in Section 5.8.2. Their other model runs entail a hot spot analysis of areas anticipated to contain maximum impacts, which used a higher density receptor placement. The Department notes that Donlin previously provided a single plume stack sensitivity analysis for the primary power plant engines and full receptor grid in support of their application for AQ0934CPT01. This sensitivity analysis offered consistent estimated results with only marginal variation in the maximum impacts. The Department finds that it remains representative noting a lack of meaningful revision to the modeled approach associated with said permit.

Model Selection

There are a number of air dispersion models available to applicants and regulators. EPA lists these models in their Guideline on Air Quality Models (Guideline), which the Department has adopted by reference in 18 AAC 50.040(f). Donlin used EPA’s AERMOD Modeling System (AERMOD) for their ambient analysis. AERMOD is an appropriate modeling system for this permit application.

The AERMOD Modeling System consists of three major components: AERMAP, used to process terrain data and develop elevations for the receptor grid and EUs; AERMET, used to process the meteorological data; and the AERMOD dispersion model, used to estimate the ambient pollutant concentrations. Donlin used the most current version of each component at the time of their application submission: AERMAP version 18081; AERMET version 21112; and AERMOD version 21112.

Meteorological Data

AERMOD requires hourly meteorological data to estimate plume dispersion. A minimum of one-year of site-specific data, or five years of representative National Weather Service (NWS) data is required, per Section 8.3 of the Guideline. When modeling with site-specific data, the Guideline states that up to five years should be used, when available, to account for year-to-year variation in meteorological conditions.

Donlin used one year of site-specific data collected at their ‘Camp’ meteorological station from August, 2020 through July, 2021. They used upper air data from the nearest NWS upper air station in McGrath, Alaska, which is appropriate for the proposed project. Donlin supplemented their surface data with concurrent NWS cloud cover data from Sleetmute, Alaska. Missing cloud cover data from Sleetmute was substituted with NWS cloud cover data from Aniak, Alaska.

The Department previously approved Donlin’s QAPP for the Camp meteorological station and provided findings on associated data. It approved the former approach during the pre-application review for AQ0934CPT01 as summarized below. Donlin provided a regional cloud cover analysis on 23 October, 2013. The Department supplemented Donlin‘s analysis with its own review of regional cloud cover data. It subsequently determined that the Sleetmute cloud cover data would be representative of the expected cloud cover at the project site. In a 16 January, 2015 teleconference, Donlin requested the substitution of Aniak or McGrath data for missing Sleetmute data based on the findings of their October 2013 submittal. The Department provided a response on 3 February, 2015 indicating that Aniak data could be substituted for missing Sleetmute data, but cloud cover data from McGrath, or Holy Cross, could not be used. The Department continues to find this approach appropriate for AQ0934CPT02. Detail regarding the subject meteorological data is subsequently discussed.

Quality Assurance Review

Site-specific meteorological data must meet the PSD quality assurance requirements outlined in EPA’s Meteorological Monitoring Guidance for Regulatory Modeling Applications, per 18 AAC 50.215(a)(3). Donlin submitted their meteorological data for Department approval following the associated 12-month collection period. The Department has used various term contractors to review Donlin’s meteorological data on its behalf, however, the final decision regarding data acceptability remains with the Department. The site-specific meteorological parameters used in Donlin’s modeling analysis are PSD-quality.

Meteorological Data Processing

Donlin correctly processed the meteorological data using AERMET. However, the following topics warrant additional discussion.

Low Wind Speed Adjustments

Donlin used the ADJ_u* option in AERMET to adjust the surface friction velocity. EPA developed this option to correct AERMOD’s tendency to overpredict impacts under stable, low wind conditions. The ADJ_u* algorithm was initially introduced as an alternative modeling technique under both EPA and Department rule. EPA has subsequently adopted the ADJ_u* algorithm as a regulatory option in its January 2017 revision to the Guideline. The Department notes that Donlin previously used this option as currently proposed as an element of their approach in AQ0934CPT01. Additional discussion regarding the appropriateness of its proposed use is provided in the Modeling Report for that permit.

Surface Characteristics

AERMET requires the area surrounding the meteorological tower to be characterized with regard to the following three surface characteristics: noon-time albedo, Bowen ratio, and surface roughness length. EPA has provided additional guidance regarding the selection and processing of the values used to represent these surface characteristics in their AERMOD Implementation Guide. They also developed a computer program, AERSURFACE, to determine the applicable surface characteristics from the U.S. Geological Survey (USGS) National Land Cover Data (NLCD) archives.

Donlin used AERSURFACE to process 2016 NLCD data in developing the surface parameters for their Camp meteorological station. A summary of these parameters is presented in Sections 3.5.1.1 and 3.5.1.2 of Appendix D to their 29 October, 2021 application.

Coordinate System

Air quality models need to know the relative location of the EUs, applicable structures, and receptors to properly estimate ambient pollutant concentrations. Therefore, applicants must use a consistent coordinate system in their analysis. Donlin used Zone 4 of the Universal Transverse Mercator (UTM) system within the 1983 North American Datum. This is a typical approach in AERMOD assessments and is appropriate for the current project.

Terrain

Terrain features can influence plume dispersion and the resulting ambient concentration. Digitized terrain elevation data is therefore generally included in an AERMOD analysis. AERMOD’s terrain preprocess, AERMAP, utilizes the terrain data to obtain the base elevations for the modeled EUs, buildings, and receptors; and to calculate a hill height scale for each receptor.

Donlin used National Elevation Dataset (NED) files for their terrain dataset. NED is the current terrain elevation dataset provided by the USGS. Their use of NED data is therefore reasonable and appropriate.

EU Inventory

Donlin included the proposed NOx, PM, PM-10, and PM-2.5-emitting EUs in their respective AAAQS/increment demonstrations; and the proposed CO-emitting EUs in their CO AAAQS demonstrations. Detail regarding the modeled EU inventory is subsequently discussed.

General Discussion

Donlin has proposed the operation of various equipment and emissions-generating processes. EUs associated with the former may be considered within the following general categories:

- Mining Activities;

- Process and Refining;

- Power Generation;

- Boilers and Heaters;

- Incinerators;

- Emergency Equipment;

- Access Roads;

- Mobile Machinery Tailpipes; and

- Liquid Storage Tanks

A comprehensive inventory of EUs is available in Appendix B of Donlin’s 29 October, 2021 PSD application. Figure 3-7 in Appendix D of said application illustrates the relative location of sources within the modeled domain. The combustion-related sources are proposed to fire either natural gas or ultra-low sulfur diesel (ULSD). Donlin’s application materials indicate that select EUs will be dual-fuel capable.

Donlin used the OPENPIT option in AERMOD to characterize the drilling, material extraction, loading and unloading, dozing, and machinery emissions within the pit area. They designated the remaining EUs as either point, volume, or area sources. The modeled emission rates and various aspects of the applicable source characterizations are discussed in Sections 5.7 through 5.11 of this report.

Secondary Emissions

PSD applicants must include secondary emissions in their ambient demonstration in accordance with 40 CFR 52.21(k)(1). EPA defines the term under 40 CFR 52.21(b)(18) as, “emissions which would occur as a result of the construction or operation of a major stationary source… but do not come from the major stationary source…”

Construction emissions are the only secondary emissions associated with the project. Donlin calculated and summarized their construction emissions in Section 2.2.2 of Appendix D to their 29 October, 2021 PSD permit application. Their calculations indicate that construction emissions are substantially smaller than the maximum project emissions for each PSD-triggered pollutant with an air quality standard or increment. Donlin, therefore, did not provide a separate modeling analysis for their proposed construction activities since the project emissions are anticipated to offer a conservative representation of construction emissions from the project.

EU Inventory for the Increment Demonstrations

As discussed in the Background section of this report, Donlin’s project will be located within a Class II area of the South Central Alaska Intrastate Air Quality Control Region. The major source baseline dates within this region are as follows:

- 8 February, 1988 for the annual NO2 increment;

- 6 January, 1975 for the 24-hour and annual PM-10 increment;

- 20 October, 2010 for the 24-hour and annual PM-2.5 increment.

All of the project EUs are increment consuming for these pollutants/averaging periods. There are no Class II increments for the other PSD-triggered pollutants and averaging periods, i.e. GHG, one-hour NO2, eight-hour O3, one-hour CO, and eight-hour CO. Donlin included all applicable EUs in their increment demonstrations.

Emission Rates

The modeled emission rates are consistent with the emissions information provided throughout Donlin’s 29 October, 2021 PSD application. The assumed emissions are generally related to the overall throughput of the mine, which is limited by the rated capacity of the gyratory crusher (GC) and the semi-autogenous grinding (SAG) mill. Donlin assumed the maximum capacity of the GC is 5,100 tph and the SAG mill 3,303 tph. The Department is, therefore, including these assumptions as enforceable terms and conditions to protect ambient air quality.

The Department is including enforceable terms and conditions limiting the total assumed capacity of the primary power plant generator sets, EUs 1 through 12, to protect ambient air quality. Donlin’s application materials indicate that they propose to use Wärtsilä generator sets with a rated capacity of 17,076 kilowatts electric (kWe) per unit. The total rated capacity of all twelve primary power plant generator sets is approximately 205 MW. The Department rounded this value to 210 MW in developing conditional language. Its approach mitigates the potential to reopen Donlin’s permit if small changes in the installed rated capacity occur. The Department notes that its rounded value represents a two-percent increase from Donlin’s assumed rated capacity. This percentage is within the margin of compliance for the modeled pollutants and averaging periods; see the results of Donlin’s modeling Section 5.19 of this report for detail. The Department, therefore, finds that a two-percent increase in the assumed rated capacity of these EUs would not create potential violations of the AAAQS or increments.

Donlin’s modeled emission rates are consistent with those prescribed under Best Available Control Technology (BACT) requirements. The Department is, therefore, including compliance with the BACT emission rates for the modeled pollutants as enforceable terms and conditions to protect ambient air quality. Additional details regarding the modeled emission rates, along with several additional ambient air conditions related to the modeled emission rates, are provided below.

Use of Worst-Case Variables

The emission rates for dual fuel-fired EUs typically vary by fuel. Donlin provided the emission rates for both fuels, natural gas and ULSD, in Appendix B of their 29 October, 2021 PSD permit application. They selected the highest of the two emission rates for their modeling analysis.

Annual emissions from the mining and mobile source activities will change as the mine matures. Donlin estimated that the maximum annual emissions, by modeled pollutant, will occur during the following years within their proposed life of mine (LOM):

- Year 16 for PM-2.5;

- Year 19 for NOx and CO; and

- Year 20 for PM-10.

Donlin generally used the same LOM years in developing the worst-case one-hour, eight-hour and 24-hour short-term emission rates. Exceptions and additional factors used to develop the modeled annual and short-term emission rates are subsequently discussed.

Wärtsilä PM Emissions

Donlin used a PM emission rate of 0.29 grams per kilowatt hour (g/kW-hr) for the proposed Wärtsilä generator sets, EUs 1 through 12, in their modeling analysis. This value represents an emission rate for both filterable and condensable particulates while firing ULSD, and is based on vendor information form Wärtsilä. The Department finds that this assumption is sufficient to characterize the emissions from the proposed power generation units.

Blasting Emissions

EPA’s Compilation of Air Pollutant Emission Factors (AP-42) contains an equation for deriving the PM emission factor for blasting. The horizontal area of the blast is the sole variable. Donlin’s application materials indicate that the maximum blast area during the proposed life of the mine is estimated at 120,000 ft2. The Department is, therefore, including a 120,000 ft2 per blast assumption as enforceable terms and conditions to protect the PM-10 and PM-2.5 AAAQS and increments.

The emissions of NO2 and CO from blasting are dependent on the quantity of blasting agent used. Donlin application materials indicate that the maximum annual consumption of blasting agent during the proposed life of the mine is estimated at 60,000 tons per year (tpy). Similarly, the emissions from mining activities will be partially dependent on the amount of rock moved per year, which is related to the amount of blasting that will occur. The Department is, therefore, including Donlin’s 60,000 tpy of blasting agent estimate as enforceable terms and conditions to protect ambient air quality.

Annual Operational Assumptions

Donlin’s application materials indicate that the mine will operate continuously on an annual basis. They correspondingly assumed that the process, power plant, and ancillary sources will also operate continuously. However, Donlin assumed an annual operation of 500 hours per year (hr/yr) for each of EUs 29 through 37, which their application identifies as liquid fuel-fired black start and emergency use units. The Department is, therefore, including Donlin’s 500 hr/yr assumption for these units as enforceable terms and conditions to protect the annual AAAQS and increments.

Wind-Blown Emissions

Donlin included the fugitive wind-blown emissions that may occur from exposed surfaces in their PM-10 and PM-2.5 modeling analyses. The exposed surfaces that could be subject to wind erosion include the haul roads, access roads, Tailings Beach, Waste Rock Facility, Short-term Stockpile, Long-term Stockpiles West and East, and Overburden Stockpile South. Wind is also a factor in determining the quantity of particulate emissions generated during the loading and unloading of aggregates. Wind erosion occurs when the wind speed over a freshly exposed surface exceeds the threshold friction velocity for the given material. The emissions are associated with intermittent wind gusts, but the emissions are conservatively assumed to occur for the entire hour for modeling purposes.

Donlin used the procedures for industrial wind erosion in Section 13.2.5 of AP-42 to estimate the particulate emissions from wind erosion. The procedure requires fastest-mile two-minute average wind speed data, which Donlin obtained by multiplying their hourly wind speed data by 1.24. Donlin previously justified their use of a 1.24 hour-to-fastest mile scaling factor in a 9 April, 2015 e-mail provided in support of AQ0934CPT01. The Department continues to find this approach appropriate for the current project on a case-specific basis. A technical discussion and factual basis for Donlin’s approach is provided in the Modeling Report for the aforementioned permit.

Donlin used the overburden classification in Table 13.2.5-5 of AP-42, and the associated 1.02 meter per second (m/s) threshold friction velocity to represent the material handled at all exposed stock piles, which includes the Tailings Beach and waste rock facility, as well as the road surfaces. The overburden classification provides a more conservative estimate of the wind-blown emissions from road surfaces than what would have occurred if the roadbed material classification is assumed. Donlin’s use of the overburden classification for their stock piles is appropriate.

Donlin used the procedures for aggregate handling and storage piles in Section 13.2.4 of AP-42 to estimate the quantity of dust emissions generated from loading and unloading operations. This procedure requires the use of a mean wind speed, rather than the fastest-mile wind speed. Donlin used the hourly wind-speeds from their meteorological data for this calculation.

Donlin did not take credit in their PM modeling assessments for dust control at the Tailings Beach. However, they reduced the fugitive dust emissions from all unpaved roadways by 90-percent, based on the control methods described in their Fugitive Dust Control Plan, provided as Appendix E to their 29 October, 2021 PSD permit application. The Department is, therefore, including enforceable terms and conditions requiring Donlin to comply with the methods described in their Fugitive Dust Control Plan to protect the PM AAAQS/increments.

Short-term Emissions Assumptions

The modeled emission rates for the short-term AAAQS and increments should generally reflect the maximum emissions allowed during the given averaging period. However, applicants may use the annual NOx emission rate for intermittently operated EUs when modeling the one-hour probabilistic NO2 AAAQS.

Donlin used the annual NOx emissions when modeling their one-hour NO2 impacts from the black start and emergency generator-engines, EUs 29 through 37, since these units are intermittently operated. They used the maximum hourly emission rates for these EUs when modeling the short-term AAAQS and increments. They likewise used the maximum hourly emission rates for all other combustion-related EUs. The Department is including Donlin’s 500 hr/yr assumption for the intermittently operated EUs as an ambient limit to protect the one-hour NO2 AAAQS.

Donlin’s application materials indicate that up to five blasts per day may occur. For the purposes of modeling, however, they assumed all five blasts would occur within the averaging period of the subject AAAQS or increment. The Department is not imposing Donlin’s assumed blasts per day as an ambient air condition since the frequency is relatively conservative, and the Department’s limits on area per blast and quantity of blasting agent used, discussed in Section 5.7.3 of this report, is anticipated to be adequate to protect the AAAQS and increments.

Donlin assumed the process-related EUs will continuously handle 5,100 tph of material, which is the maximum throughput of the GC. They also assumed the refining-related EUs will continuously handle 3,303 tph of material, which is the maximum throughput of the SAG mill. Donlin also created an hourly emissions file for the fugitive emissions from the mine pit, Tailings Beach, and Waste Rock Facility. This allowed them to calculate and use hourly-specific wind-blown emission rates for these activities. They used the annual emission rates from the worst-case LOM years to estimate the short-term emission rates for the remaining mining and mobile source activities. The former approach is appropriate for the proposed project.

Point Source Parameters

In addition to the previously discussed emission rates, applicants must provide the stack height, diameter, location, base elevation, exhaust plume exit velocity, and exhaust temperature for each EU characterized as a point source.

The Department generally found Donlin’s assumed exhaust parameters to be consistent with the vendor information or expectations for similarly sized EUs. Their assumed stack dimensions are also reasonable. For those EUs located within a building, Donlin generally used stack heights that are slightly taller than the surrounding structure. The stack heights for all other EUs are relatively short, and similar to the heights commonly found or expected for those types of EUs. The exceptions, or items that otherwise warrant additional discussion, are subsequently discussed.

General Discussion re Horizontal/Capped Stacks

Capped stacks and horizontal releases generally lead to higher impacts in the immediate near-field than the impacts from uncapped, vertical releases. EPA has, therefore, developed an option in AERMOD that revises the release parameters as described in the AERMOD User’s Guide, for any stack identified as horizontal (POINTHOR) or capped (POINTCAP).

Donlin assumed that the Wärtsilä generator sets have vertical, uncapped releases, and that all other point source releases are capped. They used the POINTCAP option to designate and characterize the capped stacks. The Department is including enforceable terms and conditions that require Donlin to construct vertical, uncapped releases on the Wärtsilä generator sets, EUs 1 through 12, to protect the AAAQS and increments.

Wärtsilä Exhaust Stacks Additional Discussion

Donlin has indicated that the twelve Wärtsilä generator sets, EUs 1 through 12, will be housed in two identical engine halls, each containing six generator sets. They further stated:

Each engine hall will consists of six stacks (one per engine) with identical release characteristics, clustered together in a configuration of two banks of three engines each. The six stacks in each cluster will be arranged tightly together, approximately one diameter apart.

Identical plumes from stacks located this closely together would promptly merge upon release. Donlin, therefore, characterized each stack cluster as a single exhaust plume in their merged plume scenario. They used the actual release height, exhaust temperature, and exit velocity of a single stack for each merged plume, but an artificially large stack diameter so that the resulting volumetric exhaust flow would equal the total volumetric exhaust flow from all six stacks.

Donlin separately provided a single plume sensitivity analysis for this approach in support of AQ0934CPT01. The results of their analysis show that the AAAQS and increments would remain protected even when operating a single generator set in each hall. Under the single plume scenario, total emissions from the Wärtsilä engines would be approximately one-sixth of those in the merged plume scenario, though the exhaust plumes would also be less buoyant. Both scenarios provided proximate model results. Impacts from the single stack scenario did not exceed those from the merged plume scenario by more than 0.02 micrograms per cubic meter (µg/m3).

The Department approved Donlin’s merged plume approach 16 March, 2015 when it was proposed it in support of AQ0934CPT01. It continues to find this approach appropriate in the absence of meaningful revision to the proposed project following said approval.

Donlin assumed the Wärtsilä engine stacks will be 49 m tall. This assumption situates the emissions release above that of the tallest nearby structure, though it is relatively tall for typical exhaust heights of reciprocating engines. The Department is, therefore, including Donlin’s 49 m stack height assumption as enforceable permit terms and conditions to protect the AAAQS and increments. The Department notes that the 49 m assumption complies with the Good Engineering Practice stack height requirements in 18 AAC 50.045(e) – (f).

Donlin plans to recover waste heat from the Wärtsilä exhaust stacks to power a steam turbine in a combined cycle process. Donlin, therefore, used the post heat recovery exhaust temperature and exhaust flow rate, provided by the vendor, for their modeling analysis.

Open Pit Parameters

AERMOD has an open pit option for characterizing PM or gaseous emissions that occur below grade. Examples of where this option could be used include open pit mines and gravel quarries. Irregularly-shaped pit areas must be characterized by a rectangle of equal area when using the open pit option. Applicants who use this option must, therefore, provide AERMOD with the length of each side, the pit volume, and the average release height of the emissions activities within the pit, in addition to the pit location and base elevation. If warranted, the applicant may also provide an orientation angle of the pit in degrees from the North. If PM emissions are modeled, the applicant must also provide the same particle size information as needed to account for particle deposition.

Donlin used the OPENPIT option to characterize the gaseous and particulate emissions from drilling, material extraction, loading and unloading, dozing, and machinery activities that will occur within the open pit. This is a reasonable option for characterizing the below grade emission activities. Donlin appropriately assumed the same particle size used to account for particulate deposition; see Section 5.12.3 for detail.

Pit sizes change during the life of the mine. Donlin used the average pit volume and average base elevation between LOM years 16 and 20. LOM year 16 has the highest anticipated emissions of CO, NOx, and PM-2.5, and LOM year 20 has the highest anticipated emissions of PM-10. The former is a reasonable approach for modeling a constantly changing activity. Detail regarding Donlin‘s derivation of these values is presented in a 30 September, 2015 technical memorandum authored by Air Sciences, Inc. titled Model Updates – Donlin Fugitive Source Parameters.

Donlin used the weighted release height of the various activities characterized by the open pit algorithm, i.e., drilling, truck loading/unloading, equipment tailpipes, and dozing, in specifying a release height for the source. The resulting release height for LOM year 16 is 4.99 m and for LOM year 20 it is 4.85 m, which are reasonable values as proposed. Detail regarding Donlin’s derivation of the open pir release height(s) may be found in the 10 September, 2015 technical memorandum authored by Air Sciences, Inc., Donlin Fugitive Source Parameters; and a 17 August, 2015 letter from Donlin titled Response to ADEC Comments on the Modeling Protocol from Donlin Gold LLC.

Volume Source Parameters

The volume source option is frequently used to characterize fugitive emissions that have initial lateral and vertical spread near the point of release. Examples include the fugitive dust associated with construction activities or dirt roads, and wind-blown dust from storage piles. Applicants who characterize an EU or emissions activity as a volume source must provide AERMOD with the initial lateral and vertical dimensions of the volume, the release height of the emissions (volume center), and the source location and base elevation, in addition to the previously discussed emissions rate.

Donlin used the volume source option to characterize the fugitive emissions from blasting activities, haul roads, the waste rock storage facility, short- and long-term ore storage sites, the eastern long-term ore storage site, tailings dam, and overburden stockpile. The volume source option for these types of emissions activities is appropriate.

Donlin used the approach recommended by the Haul Road Workgroup of EPA/State/Local Modelers to develop the initial dimensions of each haul road segment. Detail regarding Donlin‘s derivation of the haul road parameters may be found in their 17 August, 2015 letter, Response to ADEC Comments on the Modeling Protocol from Donlin Gold LLC. Donlin also used the Haul Road Workgroup guidance to develop the initial dimensions for the stockpile plumes since a significant portion of the stockpile emissions are associated with loading and unloading operations. Donlin used appropriate parameters for each volume source.

Area Source Parameters

The area source option is frequently used to characterize ground or low level releases with no thermal or momentum plume rise. It can be used in lieu of the volume source approach to characterize the fugitive dust emissions from dirt roads. It is typically used as an alternative approach to using volume sources to characterize haul roads, especially when characterizing long road segments or scenarios where receptors are within the area source footprint. Applicants who characterize an EU or emissions activity as an area source must provide AERMOD with the length and width of the area, the release height of the emissions, and the location and base elevation, in addition to the previously discussed emissions rate. If warranted, the user may also provide an orientation angle of the rectangle in degrees from the North, and the initial vertical dimension of the area source plume.

Donlin used the area source option to characterize the proposed access road and tailings storage facility. Using the area source option for the access road is appropriate given the long road segments. Using the area source option for the tailings storage facility is also appropriate based on the discussion provided by Air Sciences, Inc. in their 10 September, 2015 technical memorandum, Donlin Fugitive Source Parameters:

For the tailings storage facility (TAILS), a completely saturated tailings material (slurry) will be transferred through a pipeline (i.e., no truck dumping activity or material stockpiling; insignificant vertical dispersion), and the only emissions expected at this source are due to wind erosion of the exposed dry surfaces. Therefore, in the absence of any mechanical activity and vertical dispersion, a surface-based (zero release height) AREA source characterization was used.

Donlin used the approach recommended by the Haul Road Workgroup of EPA/State/Local Modelers to develop the release height and initial vertical dimension for the access road. Detail regarding Donlin’s derivation of the access road parameters may be found in their 17 August, 2015 letter, Response to ADEC Comments on the Modeling Protocol from Donlin Gold LLC.

Pollutant Specific Considerations

The following pollutants warrant additional discussion.

Ambient NO2 Modeling

The emissions of NOx from combustion sources include both nitric oxide (NO) and NO2 constituents. After combustion gases exit a stack, additional NO2 can be formed due to reactions within the atmosphere. Section 4.2.3.4 of the Guideline describes a three-tiered approach for estimating the ambient concentrations of NO2 from this process, ranging from the simplest but very conservative assumption that all NO is converted to NO2, to other more complex methods.

Donlin used the Ozone Limiting Method (OLM) to estimate their ambient NO2 concentrations. OLM is an appropriate approach for estimating annual average NO2 impacts under the EPA’s Guideline.

In-Stack NO2-to-NOx Ratio

The assumed NO2-to-NOx in-stack ratio (ISR) is a variable that must be set for each NOx-emitting EU when modeling the NO2 impacts with OLM. Source-specific data should be used to define this ratio when available. When source-specific data is not available, an ISR of 0.5 may be used without justification for the purposes of modeling the one-hour NO2 impacts. According to EPA’s 1 March, 2011 one-hour NO2 modeling guidance, this value represents a reasonable upper bound based on the available in-stack data. EPA has not provided a similar ‘default’ ratio for the purposes of modeling the annual average NO2 impacts.

Donlin performed a review of literature and stack test data to characterize the ISRs for their NOx-emitting EUs. These ratios are provided in their application materials. The Department maintains its findings regarding the appropriateness of these ISRs, ceteris paribus, as discussed in its 28 September, 2015 approval of Donlin’s modeling protocol for AQ0934CPT01. The aforementioned ISRs are presented in Table 2.

Table 2. NO2-to-NOx ISRs in AQ0934CPT02

| EU Source Category | ISR |
| --- | --- |
| Blasting | 0.036 |
| Diesel Engines | 0.11 |
| Diesel Engines w/Catalyzed Particulate Filters | 0.22 |
| Diesel Boilers | 0.05 |
| Natural Gas Boilers | 0.10 |
| Diesel Machinery | 0.11 |

Ozone Data

OLM requires ambient ozone data in order to determine how much of the NO is converted to NO2. Donlin used a temporally-varying data set that they derived from their pre-construction O3 data. They developed a generic monthly-hour-of day O3 profile that was used with their corresponding meteorological data. Donlin developed the former profile by taking the multi-year average of the maximum O3 concentration for a given hour of day, i.e., hour 1 through 24, and within each month, i.e., January through December. The resulting O3 profile is presented in Table 3-15 of Appendix D to their 29 October, 2021 PSD permit application. Donlin’s approach in deriving a generic O3 data set for NO2 modeling is appropriate for the project. This approach also allows the data set to reflect significant seasonal variations that occur within Alaska.

AERMOD Settings

Donlin used the OLMGROUP ALL setting within AERMOD in their NO2 modeling analysis. This setting is consistent with EPA’s 1 March, 2011 one-hour NO2 modeling guidance.

PM-2.5

PM-2.5 may be directly emitted from a source and is also formed through chemical reactions in the atmosphere, i.e. by secondary formation with other pollutants. AERMOD is an acceptable model for performing a near-field analysis of the direct emissions, but EPA has not developed a near-field model that includes the necessary chemistry algorithms for estimating secondary impacts. EPA, therefore, recommends that applicants use existing technical information to assess their secondary PM-2.5 impacts by way of a “Tier 1” analysis. The use of photochemical modeling to assess secondary impacts, i.e. a “Tier 2” analysis, may be appropriate as warranted.

Donlin’s direct PM-2.5 and NOx precursor emissions exceed the respective PSD significant emission rates (SERs). In this situation, EPA recommends the use of air quality modeling to assess the direct impacts and states that one of the following options could be used for assessing the secondary impacts: a qualitative approach, a hybrid qualitative and quantitative approach that utilizes existing technical work, or a full quantitative photochemical grid modeling analysis. Of the three options for assessing secondary impacts, EPA stated that “only a few situations would require explicit photochemical grid modeling”, i.e., the photochemical modeling approach would rarely be warranted.

Donlin used a qualitative approach to assess their secondary PM-2.5 impacts. This is an appropriate approach for stationary sources located in rural Alaska, or other areas with limited area-wide precursor emissions.

EPA has issued guidance regarding the characterization of secondary formation in various PSD scenarios.. In its guidance, EPA notes that the maximum direct impacts and the maximum secondary impacts from a stationary source “…are not likely well-correlated in time or space”, i.e., they will likely occur in different locations and at different times. This difference occurs because secondary PM-2.5 formation is a complex photochemical process that requires the presence of precursor pollutants in sufficient quantity for significant formation to occur. The conditions for this reaction process to meaningfully occur within the immediate project near-field, the location of maximum project impacts, is not anticipated to be likely.

EPA further stated that representative ambient monitoring data could be used to address the secondary formation that occurs from existing sources in a demonstration of the ambient standard. Donlin’s pre-construction data used to represent the background concentration in their PM-2.5 AAAQS analysis is sufficient for requirement on a case-specific basis; see Section 5.17 for detail.

The Department notes that Donlin’s qualitative PM-2.5 analysis indicates that the measured 24-hour and annual PM-2.5 concentrations are below the respective AAAQS. There appears to be no indication that secondary PM-2.5 formation from existing sources are causing or contributing to violations of the PM-2.5 AAAQS. Donlin also provided additional arguments that appropriately demonstrate that the PM-2.5 AAAQS will not be threatened by secondary PM-2.5 formation. Donlin’s discussion of temporal and spatial factors of influence were similarly advanced in support of their PM-2.5 AAAQS and increment demonstrations. The resultant PM-2.5 modeling analyses illustrate a substantial margin of compliance with the 24-hour and annual PM-2.5 increments, as presented in Section 5.19, and that the minimal impacts from secondary formation at the maximum impact locations could be accommodated without potential violations of the Class II increments. Donlin did not include the effects of secondary PM-2.5 formation from area-wide sources in their PM-2.5 increment demonstrations since the regional sources are not increment consuming; see Section 3.6 for detail.

Particle Deposition

Deposition refers to the natural settling of particles that occurs as a PM plume travels downwind. AERMOD has two algorithms for simulating this occurrence: Method 1 and Method 2. The Method 1 approach may be applied under the regulatory default option of AERMOD, i.e. the use of Method 1 is allowed in a regulatory modeling analysis. The Method 2 approach is considered a non-Guideline method and, therefore, requires case-specific approval from the Department and EPA under the alternative modeling procedures of the Guideline. Donlin used the Method 1 deposition option within AERMOD to improve the accuracy of their estimated PM-10 and PM-2.5 concentrations.

The Method 1 algorithm requires data that reflects the particle size distribution for each activity with PM emissions. The user categorizes the emissions by particle size and then provides AERMOD the mass-mean aerodynamic particle diameter, mass fraction, and particle density for each category. Donlin calculated and categorized their particulate emissions as indicated in Tables 3-18 through 3-20 of Appendix D to their 29 October 2021 PSD permit application. Donlin’s approach is appropriate to incorporate the effects of particle deposition in AQ0934CPT02.

Downwash

Downwash refers to the situation where local structures influence the plume from an exhaust stack. Downwash can occur when a stack height is less than a height derived by a procedure called Good Engineering Practice (GEP), which is defined in 18 AAC 50.990(42). It is a consideration when there are receptors relatively near the applicant’s structures and exhaust stacks.

EPA developed the Building Profile Input Program - PRIME (BPIPPRM) program to determine which stacks could be influenced by nearby structures and to generate the cross-sectional profiles needed by AERMOD to determine the resulting downwash. Donlin used the current version of BPIPPRM, version 04274, to determine the building profiles needed by AERMOD.

Donlin included all of the modeled point sources in their downwash analysis. The Department used a proprietary 3-D visualization program to review their characterization of the exhaust stacks and structures. The characterization matches the figures provided in their permit application. Donlin appropriately accounted for downwash in their modeling analysis. BPIPPRM indicated that the modeled exhaust stacks are within the GEP stack height requirements.

Ambient Air Boundary

The AAAQS only apply within location of ambient air, which has been defined by EPA as “…that portion of the atmosphere, external to buildings, to which the general public has access.”  Applicants may, therefore, exclude areas that they own or lease from an ambient demonstration if they employ “…measures, which may include physical barriers, that are effective in precluding access to the land by the general public.”  They conversely need to model that portion of their property/lease that has no such restriction, or where there is an easement or public right-of-way. Natural features, such as dense vegetation or topographical features, can provide adequate barriers to public access, although the adequacy of the given features must be evaluated on a case-specific basis.

Donlin identified a Core Operating Area (COA) to indicate the area(s) where public access will be precluded. They used the COA boundary as their ambient air boundary. The COA is identified in figures throughout Appendix D of their 29 October, 2021 PSD application and is discussed in their public access control plan (PACB), initially developed in March of 2017 and provided with their application materials.

Donlin asserts that their lease agreements grant them the legal authority to preclude access within the COA. They provided letters from both The Kuskokwim Corporation (TKC), and the Calista Corporation (Calista), that confirms they have the authority to preclude public access within the portion of the COA that is owned by the given corporation. They provided an abbreviated copy of the lease agreement with Lyman Resources in Alaska, Inc. (Lyman) on 19 January, 2016 confirming that they have exclusive access and use of the leased surface lands.

Donlin notes that there are 15 publicly recognized access easements or right-of-ways within the COA that they are attempting to reroute. They previously indicated that they petitioned the Alaska Department of Natural Resources, the Alaska Department of Transportation and Public Facilities, and the U.S. Bureau of Land Management to extinguish the easements within the COA. The Department is including enforceable terms and conditions to protect ambient air quality prohibiting operation until all easements or rights-of-way within the COA have been either extinguished or rerouted to areas outside the COA.

Donlin will use the methods described in their PACB to restrict public access within the COA. The methods vary by location, but may include fencing/gates, natural barriers, and surveillance by mine security. Donlin proposes to annually inform TKC and Calista shareholders on the access restrictions, as well as place warning signs at strategic locations. The Department is appending the PACB as a permit attachment and has included a condition requiring Donlin to restrict public access within the COA as described in the PACB.

Worker Housing

Donlin will need to house their workers on site due to the project’s remote location. Worker housing areas must be treated as ambient air, except under the conditions described in the Department’s Ambient Air Quality Issues at Worker Housing policy. The conditions are:

- the worker housing area is located within a secure or remote site;

- the worker housing area is for official business/worker use only; and

- the operator has a written policy stating that the on-site workers are on 24-hour call.

Donlin did not consider their worker housing area as a part of ambient air for the bases described in Section 3.4.2 of Appendix D to their 29 October, 2021 PSD permit application. The Department finds that their proposed worker housing meets the conditions listed in aforementioned policy.

Receptor Grid

Donlin used 50 m receptor grid spacing along their ambient air boundary. They placed additional receptors beyond the 50 m boundary grid as follows:

- every 100 m within the first 500 m of the 50 m grid; and

- every 500 m within the next km beyond the 100 m grid.

Following the model runs that observe the former receptor placement, Donlin confirmed their maximum impacts by performing a fine grid hot spot analysis for each pollutant and averaging period. The fine receptor grid consisted of receptors placed every 25 m within the general area of maximum impact for a given pollutant and averaging period. The hot spot analyses were found to offer proximate results. This approach both reduces uncertainty in the model-estimated values and demonstrates that the receptor grid has sufficient resolution to determine the maximum impacts.

Off-Site Impacts

The air quality impact from natural and regional sources, along with long-range transport from far away sources, must be accounted for in a cumulative AAAQS demonstration. The approach for incorporating these impacts must be evaluated on a case-specific basis for each type of assessment and for each pollutant, as applicable

Section 8.3 of the Guideline discusses how the off-site impacts could be incorporated for purposes of demonstrating compliance with an air quality standard. These impacts must be represented through either ambient monitoring data or through modeling. However, Section 8.3.3(b)(iii) notes, “The number of nearby sources to be explicitly modeled in the air quality analysis is expected to be few except in unusual situations.” The language in this section further states that “…sources that cause a significant concentration gradient in the vicinity of the [applicant’s source] are not likely to be adequately characterized by the monitored data due to the high degree of variability of the source’s impacts.”

Donlin’s project is located in a remote part of Alaska. There are no nearby stationary sources that would cause a significant concentration gradient within the project area. Illustratively, the nearest stationary source with an air quality control operating permit, the Bethel Power Plant, is approximately 250 km from the project area. This distance is beyond the 50 km range of AERMOD. Donlin, therefore, used their pre-construction monitoring data to represent the impact from all natural and anthropogenic off-site sources in their cumulative AAAQS analysis. This approach is both appropriate for the current project and consistent with the recommendations of EPA’s Guideline.

There are various ways to add a background concentration to the modeled concentration in a cumulative AERMOD analysis. The typical practice is to manually sum the two values. Recent versions of AERMOD, however, include an option where the background concentration can be automatically added to the modeled concentration. This approach also allows applicants to observe temporarily varying background concentrations in their ambient demonstrations.

In their CO, PM-10, and PM-2.5 AAAQS demonstrations, Donlin added the maximum monitored concentration, in accordance with the form of the relevant AAAQS, to their modeled design concentration. They used temporarily varying background concentrations for their one-hour and annual NO2 AAAQS demonstrations. Donlin did not include background concentrations in their cumulative increment assessments since there are no increment consuming sources within the project area.

The Department notes that Donlin derived temporally varying NO2 background concentrations using the same approach as that for their O3 profile; see Section 5.12.1.2 for detail. The resultant NO2 concentrations are presented in Table 3-17 of Appendix D to their 29 October, 2021 PSD permit application. Donlin’s use of the multi-year average of the maximum hour of day values within the month is consistent with EPA’s one-hour NO2 modeling guidance.

Design Concentrations

EPA allows applicants to use modeled concentrations that are consistent with the form of the given standard or increment. Donlin generally used the modeled concentrations that are consistent with this approach. They used a more conservative approach in their 24-hour PM-10 AAAQS and annual PM-2.5 AAAQS demonstrations. Donlin’s design concentrations are summarized in Table 3.

Table 3. Modeled Design Concentrations in AQ0934CPT02

| Pollutant | Avg. Period | AAAQS | Class II Increment |
| --- | --- | --- | --- |
| NO2 | One-hour | h8h | n/a |
|  | Annual | HY | HY |
| PM-10 | 24-hour | h2h | h2h |
|  | Annual | n/a | HY |
| PM-2.5 | 24-hour | h8h | h2h |
|  | Annual | HY | HY |
| CO | One-hour | h2h | n/a |
|  | Eight-hour | h2h | n/a |

Table Notes:

h2h: high, second-high.

h8h: high, eighth-high. For purposes of 1-hour NO2, the “h8h” is the five-year average of the high, eighth-high of the daily maximum 1-hr NO2 concentrations. For purposes of 24-hour PM-2.5, the “h8h” is the five-year average of the high, eighth-high of the 24-hour PM-2.5 concentrations.

HY: highest annual average from any year.

Results and Discussion

The maximum NO2, CO, PM-10, and PM-2.5 impacts from Donlin’s AAAQS demonstration are presented in Table 4, along with the background concentrations, total impacts, and respective AAAQS. The total impact is less than the AAAQS for each pollutant and averaging period. Therefore, Donlin has demonstrated compliance with the NO2, CO, PM-10 and PM-2.5 AAAQS.

Table 4. Maximum Impacts Compared to the AAAQS

| Pollutant | Avg. Period | Max. Conc. (g/m3) | Bkgd. Conc. (g/m3) | Total Impact (g/m3) | AAAQS (g/m3) |
| --- | --- | --- | --- | --- | --- |
| NO2 | One-hour | 119 | See Notes | 119 | 188 |
|  | Annual | 11.6 | See Notes | 11.6 | 100 |
| PM-2.5 | 24-hour | 3.2 | 6.8 | 10 | 35 |
|  | Annual | 0.6 | 2.3 | 2.9 | 12 |
| PM-10 | 24-hour | 21.6 | 14.1 | 35.7 | 150 |
| CO | One-hour | 10,582.1 | 686.9 | 11,269 | 40,000 |
|  | Eight-hour | 2,679.1 | 457.9 | 3,137 | 10,000 |

Table Notes:

The NO2 background concentration is included as a part of the AERMOD run.

The impacts from Donlin’s NO2, PM-10, and PM-2.5 Class II increment demonstrations are presented in Table 5, along with the respective Class II increment. The maximum impact is less than the applicable Class II increment for each pollutant and averaging period. Therefore, Donlin has demonstrated compliance with the NO2, PM-10, and PM-2.5 Class II increments.

Table 5. Maximum Impacts Compared to the Class II Increments

| Pollutant | Avg. Period | Max. Modeled Conc. (g/m3) | Class II Increment (g/m3) |
| --- | --- | --- | --- |
| NO2 | Annual | 4.5 | 25 |
| PM-2.5 | 24-hr | 7.5 | 9 |
|  | Annual | 0.6 | 4 |
| PM-10 | 24-hr | 21.6 | 30 |
|  | Annual | 4.3 | 17 |

OZONE IMPACTS

As discussed in the Background section, VOC is a triggered PSD-pollutant for this project. There is no VOC AAAQS, but VOC and NOx emissions can form O3, which does have an AAAQS. Donlin was, therefore, required to demonstrate compliance with the O3 AAAQS, in accordance with 40 CFR 52.21(k).

O3 is not usually emitted directly into the air. It is instead created in the atmosphere through chemical reactions involving sunlight and NOx/VOC emissions. It is inherently a regional pollutant, the result of chemical reactions between emissions from many NOx and VOC sources over a period of hours or days, and over a large area. EPA’s Guideline does prescribe a recommended model for assessing the O3 impact from an individual stationary source. Qualitative approaches are generally used to meet the 40 CFR 52.21(k) ambient demonstration requirement.

Donlin’s project is located in an area that is designated as unclassifiable for all criteria pollutants, including O3. There are no O3 non-attainment areas in Alaska, including those areas with greater emissions of NOx and VOC. It is, therefore, unlikely that the NOx and VOC emissions from Donlin’s project would cause or contribute to a violation of the O3 AAAQS. Donlin provided a comparison of the project emissions to a larger selection of NOx and VOC emissions from Anchorage to support the former assumption. They then noted that, while reported Anchorage emissions are 4 to 10 times higher than those from their project, the ambient concentration still complies with the O3 AAAQS. The Department finds that Donlin’s O3 demonstration is appropriate for AQ0934CPT02.

A summary of the emissions and concentrations that Donlin compared are presented in Table 6. Detail regarding their comparison is found in Section 3.13.3 of Appendix D to their 29 October, 2021 PSD permit application.

Table 6. DGP and Anchorage Area O3 Comparison

| Source | O3 Precursor Emissions (tpy) |  |  | Monitored 8-hr O3 Conc. (ppm) | 8-hr O3 AAAQS (ppm) |
| --- | --- | --- | --- | --- | --- |
|  | NOx | VOC | Total |  |  |
| DGP | 3,258 | 1,279 | 4,537 | 0.051 | 0.070 |
| Anchorage Area | 12,298 | 14,428 | 26,726 | 0.045 |  |

ADDITIONAL IMPACT ANALYSES

PSD applicants must assess the impact from the proposed project and associated growth on visibility, soils, and vegetation per 40 CFR 52.21(o). Donlin provided the additional impact analysis in Section 3.13.4 of Appendix D to their 29 October, 2021 PSD permit application. The Department’s findings regarding their additional impact analysis are subsequently discussed.

Visibility Impacts

PSD applicants must assess whether the emissions from their stationary source, including associated growth, will impair visibility. Visibility impairment means any humanly perceptible change in visibility, such as visual range, contrast, or coloration, from that which would have existed under natural conditions. Visibility impacts can occur as visible plumes, i.e., plume blight, or in a general, area-wide reduction in visibility, also known as regional haze. Alaska does not have standards for plume blight. For Class I areas, the Federal Land Manager (FLM) provides the desired thresholds. There are no established thresholds for Class II areas. The typical tool for assessing plume blight is EPA’s VISCREEN model.

The maximum range of VISCREEN is 50 km. When Class I areas lie beyond that range, as is the case for the current project, the Department recommends that applicants use the 50 km maximum range as the source to observer distance. This approach provides an upper bound of the potential plume blight impacts at more distant locations. In Donlin’s case, using a 50 km source to observer distance provides very conservative results noting that Denali is approximately 315 km away. When running VISCREEN in an upper bound analysis, the 50 km range would also be used as the nearest source to boundary distance per page 24 of EPA’s Workbook for Plume Visual Impact Screening and Analysis (Revised).

Since there are no Class II visibility thresholds, VISCREEN compares the visibility impacts to the Class I thresholds. VISCREEN provides results for impacts located inside a Class I area and for impacts located outside a Class I area. The latter is used in situations where there is an integral vista. In situations where there are no integral vistas, applicants only need to use the results for impacts located inside a Class I area. Alaska only has two integral vistas, both of which are associated with the Denali Class I area. Since the integral vistas are beyond the 50 km range of VISCREEN, the Department informed Donlin that they only needed to report the ‘inside’ results.

Donlin used the current version of VISCREEN, 13190, to estimate their worst-case plume blight. They appropriately assumed an ozone concentration of 40 parts per billion (ppb) and a background visual range of 250 km. They also appropriately excluded the fugitive and mobile emissions from the plume blight analysis since those emissions do not consist of coherent plumes. Donlin, therefore, observed the annual NOx and PM emissions from their point sources.

Donlin’s initially used the default “Level 1” approach of assuming a constant 1.0 m/s wind speed and extremely stable ‘F’ class atmospheric conditions. This approach showed potential plume blight at 50 km. The Department notes that while Donlin appropriately followed standard practice in their Level 1 analysis, the results are very conservative and unlikely. Illustratively, winds would need to remain steady during the 87.5 hours it would take for a plume to travel from Donlin to Denali at 1.0 m/s.

Donlin’s subsequently performed a “Level 2” analysis, which relies on more realistic plume travel times and uses site-specific meteorological conditions. EPA’s Workbook for Plume Visual Impact Screening and Analysis (Revised), states: “For the Level-2 screening analysis, we assume it is unlikely that steady-state plume conditions will persists for more than 12 hours.” The wind speed would therefore need to be 8 m/s or greater for the plume to travel 315 km within 12 hours.

Soil and Vegetation Impacts

The ambient demonstration provided by applicants is typically adequate to show that their air emissions will not cause adverse impacts to soil or vegetation. EPA has established what they refer as secondary NAAQS in order to protect public welfare. The term ‘welfare’ is defined in Section 302(h) of the Clean Air Act to include “effects on soils, water, crops, vegetation ...” The AAAQS and primary NAAQS are identical for each of the modeled pollutants. However, the annual PM-2.5 secondary NAAQS (15 µg/m3) is less stringent than the annual PM-2.5 primary NAAQS/AAAQS (12 µg/m3). Therefore, a modeling analysis that demonstrates compliance with the AAAQS also demonstrates compliance with the secondary NAAQS.

Donlin demonstrated that they can comply with the AAAQS. Therefore, their ambient analysis indicates that they will not cause adverse impacts to soil or vegetation. The maximum cumulative impacts for the PSD-triggered pollutants with secondary NAAQS are presented in Table 7.

Table 7. Maximum Total Impacts Compared to the Secondary NAAQS

| Pollutant | Avg. Period | Total Impact (g/m3) | Secondary NAAQS (g/m3) |
| --- | --- | --- | --- |
| NO2 | Annual | 11.6 | 100 |
| PM-2.5 | 24-hour | 10 | 35 |
|  | Annual | 2.9 | 15 |
| PM-10 | 24-hour | 35.7 | 150 |

Associated Growth Analysis

40 CFR 52.21(o)(2) requires PSD applicants to provide an analysis of the air quality impact projected for the area as a result of general commercial, residential, industrial, and other growth associated with the source or modification. Donlin does not anticipate significant changes in these aspects of concern from their proposed project, which suggests there would be no associated impact on air quality. The Department finds Donlin’s assessment reasonable.

CONCLUSION

The Department reviewed Donlin’s permit application and concluded the following:

- Donlin’s ambient demonstration satisfies the Source Impact Analysis requirements of 40 CFR 52.21(k). Donlin demonstrated that the NOx, PM-10, PM-2.5, CO, and VOC emissions associated with operating the stationary source, within the restrictions listed in this report, will not cause or contribute to a violation of the NO2, PM-10, PM-2.5, CO and O3 AAAQS. They also demonstrated that the emissions will not cause or contribute to a violation of the NO2, PM-10, and PM-2.5 Class II increments.

- Donlin appropriately used the models and methods required under 40 CFR 52.21(l) Air Quality Models.

- Donlin conducted their modeling analysis in a manner consistent with the Guideline, as required under 18 AAC 50.215(b)(1).

- Donlin pre-construction data satisfies the Preapplication Analysis requirements of 40 CFR 52.21(m)(1).

- Donlin adequately addressed the Additional Impact Analysis provisions in 40 CFR 52.21(o).

The Department developed conditions in Construction Permit AQ0934CPT02 to ensure that Donlin complies with the modeled AAAQS and Class II increments. These conditions are summarized as follows:

To protect the NO2, PM-10, PM-2.5, and CO AAAQS, and the NO2, PM-10, and PM-2.5 Class II increments, the Permittee shall:

- Not operate under the Construction Permit until all easements or rights-of-way within the COA have been either extinguished or rerouted to areas outside the COA;

- Limit the throughput of the gyratory crusher to 5,100 tph and the semi-autogenous grinding mill to 3,303 tph;

- Limit the total rated capacity of the power plant generator sets, EUs 1 through 12, to no more than 210 MWe;

- Comply with the emissions standards for EUs subject to BACT;

- Use no more than 60,000 tpy of blasting agent at the stationary source;

- construct vertical, uncapped exhaust releases on the power plant generator sets, EUs 1 through 12; and

- construct exhaust releases for the power plant generator sets, EUs 1 through 12, that are at least 49 m in height.

To protect the PM-10 and PM-2.5 AAAQS and Class II increments, the Permittee shall:

- Limit the size of blasting area to no greater than 120,000 ft2 per blast; and

- Comply with the Best Practical Methods (BPMs) described in the Fugitive Dust Control Plan

To protect the annually averaged NO2 and PM-2.5 AAAQS, the annually averaged NO2, PM-10, and PM-2.5 Class II increments, and the one-hour NO2 AAAQS, the Permittee shall:

- Limit the emissions from intermittently used EUs 29 through 37 to no greater than 500 hr/yr.

There are no ambient demonstration requirements for GHG emissions since there are no GHG AAAQS or increments.

In accordance with the requirements of 40 CFR 52.21(p).

E-mail from D. Jones (Department) to A. Stacey, C. Collins, et. al. (NPS); Donlin Gold LLC, Donlin Gold Project PSD Construction Permit Application & 3rd PSD Construction Extension Request; 23 November, 2021.

As approved with comment on 28 September, 2015.

Salient aspects addressed in the modeling protocol for AQ0934CPT01, and in subsequent pre-application discussions, include updates to fugitive source parameters and receptors, and a request to use the U.S. EPA’s ADJ_U* surface friction velocity parameter. ADJ_u* transitioned from an alternative modeling technique to a regulatory option in EPA’s 17 January, 2017 revision to the Guideline on Air Quality Models.

The Department’s minor source baseline dates for PSD increments are established in accordance with 40 CFR 52.21(b)(14)(ii), which the Department has adopted by reference in 18 AAC 50.040(h). A narrative of the PSD increment concept and associated baseline dates may be found in the October 20, 2010 Federal Register notice for the PM-2.5 increment program (Pages 64864 – 64907 of Federal Register Volume 75, No. 202).

EPA has the authority under 40 CFR 52.21(m)(1)(ii) to require pre-construction monitoring for PSD-triggered pollutants that do not have a NAAQS (when they have shown a need for the data), but they have not made this determination for those pollutants.

The District of Columbia Circuit Court of Appeals vacated the PM-2.5 SMC on January 22, 2013. Therefore, projects that trigger PSD review for PM-2.5 must include pre-construction monitoring data, regardless of the project impacts.

Donlin provided this analysis in a 23 October, 2013 e-mail from N. Enos (Donlin) to A. Schuler (Department); RE: ADEC Answer to Donlin Cloud Cover Question. They included a 10 October, 2013 technical memorandum from Air Sciences, Relevance of Ceiling Height and Cloud Cover in AERMET/AERMOD, as an attachment.

The Department informed Donlin of its decision to accept Sleetmute cloud cover data in an 10 October, 2013 e-mail from Alan Schuler (Department) to Nick Enos (Donlin); Donlin May Use Sleetmute Cloud Cover Data.

E-mail from James Renovatio (Department) to Mike Rieser (Donlin); FW: Cloud cover for Donlin; 3 February, 2015.

EPA proposed adopting the ADJ_u* algorithm as a regulatory modeling option as part of a 29 July, 2015 proposal to revise the Guideline. EPA also proposed a second modeling option, ‘LOWWIND3’, that could be used with or without the ADJ_u* option, to further mitigate the low wind speed issue in AERMOD. Donlin did not request to use this option, nor did they use the LOWWIND3 option in their ambient demonstration.

The rated capacity of these duel fuel units is indicated as 16,786 kWe when firing ULSD and 17,076 kWe when firing natural gas.

The Department notes that other mining activities may be reduced during periods of blasting, especially large blasts, for safety or operational purposes. Donlin’s approach of using the maximum blast area is anticipated to be conservative.

Donlin did not specify an assumed LOM year in which the most blasting agent would occur. Since other mining activity emissions are anticipated to be lower during the years of frequent blasting years, Donlin’s use of the maximum NO2 and CO blasting emissions, ceteris paribus, offers a conservative approach.

E-mail from M. Rieser (Donlin) to A. Schuler (Department); RE: Fastest Mile Wind Speed; 9 April, 2015.

I.e. best practical methods (BPMs).

EPA Memorandum from Tyler Fox to Regional Air Division Directors, Additional Clarification Regarding Application of Appendix W Modeling Guidance for the 1-hour NO2 National Ambient Air Quality Standard; March 1, 2011.

Illustratively, Donlin conservatively assumed all blasts would occur within an hour when modeling their one-hour impacts; within an eight-hour period when modeling the eight-hour impacts; and within a 24-hour period when modeling the 24-hour impacts.

User’s Guide for the AMS/EPA Regulatory Model (AERMOD); EPA-454/B-16-011; December, 2016.

E-mail from A. Schuler (Department) to M. Rieser (Donlin); ADEC Okays Donlin’s Merged Plume Proposal; 16 March, 2015.

The modeling files indicate that the Mill is the tallest structure near the Wärtsilä engine stacks. These files also indicate that the tallest part of the Mill is 34.6 m high. The resulting stack-height to building-height ratio is 1.4.

Donlin provided this technical memorandum as an attachment to a 10 September, 2015 e-mail from M. Rieser (Donlin) to A. Schuler (Department); RE: Quick Questions re Donlin’s u* Sensitivity Analysis.

Memorandum from R. Robinson, EPA Region 5 and M. Daye, EPA Region 7 to T. Fox, Haul Road Workgroup Final Report; 6 December, 2011.

The emissions of NOx, SO2, VOC, and Ozone are considered precursor emissions.

EPA’s tiered approach to assessing secondary PM-2.5 formation is described in Section 5.4 of the Guideline.

Guidance for PM2.5 Permit Modeling (EPA-454/B-14-001); May 2014.

The term “ambient air” is defined in 40 CFR 50.1. The Alaska Legislature has also adopted the definition by reference in AS 46.14.990(2).

EPA has authored multiple guidance documents regarding ambient air issues which may be found in their Modeling Clearinghouse Information Storage and Retrieval System at http://cfpub.epa.gov/oarweb/MCHISRS/. This language originates from the 2 December, 2019 Memorandum from EPA Administrator Andrew R. Wheeler to Regional Administrators: Revised Policy on Exclusions from ‘Ambient Air.

ADEC Policy and Procedure 04.02.108: Worker Housing Aggregation and Modeling, 5 May, 2021.

Workbook for Plume Visual Impact Screening and Analysis (Revised), (EPA-454/R-92-023); October 1992.