# Safety of Processing Water and Ice

Monitoring Frequency: Daily

Controls

- Water used on vessel is clean seawater.

- Water intake is above the discharge port or located on the opposite side of the vessel.

- Ice is from permitted source.

- Chlorine residual in processing water tested using appropriate test strips and ppm recorded.

Corrections

a.& b. In the event of a seawater contamination issue, processing will stop production and resume only after location has changed or contamination event is no longer present.

- Ice is only from an approved source.

- If no chlorine residual in processing water, processing will stop production and resume only after corrected and residual chlorine is detected on the test strip.

# Condition and Cleanliness of Food Contact Surfaces

Controls and Monitoring

- Food contact surfaces are cleanable and in good condition. Sanitation supervisor inspects surfaces to ensure they are cleanable. Monitoring Frequency: Daily

- Food-contact surfaces are cleaned and sanitized:

  - Before work begins, food-contact surfaces are rinsed with water, then sanitized with a 50-200 ppm chlorine bleach sanitizer. The sanitation supervisor inspects food contact surfaces to determine if they are sanitized. Monitoring Frequency: Daily before operations.

  - After work, fish waste is removed from work area. Surfaces are rinsed with cold water. Food-contact surfaces are sanitized with a 50-200 ppm bleach solution.

  - Utensils are cleaned, rinsed in hot water, and wiped/immersed in a 50-200 ppm bleach solution. Monitoring Frequency: After processing.

- Crew wear clean gloves and outer garments

- Waterproof garments/gloves are cleaned/sanitized at the beginning and end of work.

Corrections

- Food-contact surfaces that are not cleanable are repaired or replaced.

- Unclean food-contact surfaces that are cleaned and sanitized or replaced.

- Sanitizer concentrations are verified with test strips to be in range:

  - Sanitizer concentrations that are too low could result in insufficient sanitization.

  - Sanitizer concentrations that are too high could result in adulteration of the product.

# Prevention of Cross-Contamination

Controls and Monitoring

- Crew practices do not contaminate fish product:

  - Wear gloves and clean or replace as needed.

  - Wash hands and gloves thoroughly and sanitize before each work session.

  - Do not eat food or use tobacco in processing area.

  - Sanitize boots in a bath containing 100-200 ppm bleach sanitizer.

  - Production supervisor monitors crew practices.

- Boot sanitizing solutions are checked before use. Monitoring frequency: Daily before operations.

- Fish waste is removed from processing area after processing. Monitoring frequency: After processing.

- Floors are sloped to facilitate drainage. Monitoring frequency: Daily before operations.

Corrections

- Production supervisor has sanitation training.

- Crew corrects any problems with cleanliness or sanitation.

- Boot sanitizing solution is changed.

- Residual fish waste is removed.

- Floor drains are unplugged and draining adequately.

# Hand Washing/Sanitizing and Toilet

Monitoring frequency: Daily before operations.

Controls

- Toilet facilities are clean and in working order.

- Hand washing/sanitizing facilities are provided in processing areas. Hot and cold water, soap, and paper towels are provided.

Corrections

- Sanitation supervisor cleans toilet facility.

- Sanitation supervisor restocks facilities.

# Protection of Food, Food Packaging Material, and Food-Contact Surfaces from Chemical, Physical, and Biological Hazards

Monitoring frequency: Daily before operations

Controls

- Only food grade hoses used to deliver water used for processing and food contact surfaces.

- All cleaning chemicals are safe for use with food.

- Food, packaging materials and food-contact surfaces are protected from contamination from biological, chemical, and physical sources such as drips or condensate.

- Equipment is in good repair.

Corrections

- Non-food grade hoses replaced with food grade hoses.

- Unapproved chemicals are returned or used in non-processing areas.

- Safety of product is determined.

- Repairs are made as needed.

# Labeling, Storage and Use of Toxic Compounds

Monitoring frequency: Daily before operations.

Controls

- Cleaning chemicals, fuels, pesticides, and lubricants are labeled and stored away from the processing area.

- Food-grade and non-food-grade chemicals are not stored together.

Corrections

- Toxic compounds without proper info are not used until proper info is obtained.

- Improperly stored chemicals are put in correct area.

# Employee Health

Monitoring Frequency: Daily before operations.

Controls

a. Crew does not show signs of health problems that could compromise sanitation.

Corrections

a. Crew posing a risk are sent home or reassigned.

# Pests

Monitoring frequency: Daily before and during operations.

Controls

- The processing area is adequately protected from the elements and pests.

- No dogs or other animals allowed on the vessel.

- Pest control devices working and clean.

Corrections

- Adequate protection is installed.

- Dogs and other animals are removed from the vessel or kept way from the processing, packaging, and storage areas.

- Pest control devices are cleaned regularly.